高压清洗机说明书

High pressure washer manual

YLQ 7711E-200/250

未经阅读使用说明书不得操作清洗机

WARNING: Do not use the appliance without reading the instruction sheet

上海亿力电器有限公司

Shanghai Yili Electric Co., Ltd.

整机结构图

machine structure drawing

整机明细表

BOM

| 30 | 滚轮轴 wheel shaft | 2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 29 | 空心滚轮 hollow wheel | 2 |  |  |  |
| 28 | 2_5开口销 uncork | 2 |  |  |  |
| 27 | 六角法兰面螺栓加大系列B级-M8X60 | 4 |  |  |  |
| 26 | 机架管组件 lower pipe | 2 |  |  |  |
| 25 | 非金属嵌件六角法兰面锁紧螺母-M8 GB-T6183 | 9 |  |  |  |
| 24 | 1型非金属嵌件六角锁紧螺母-M6 GB-T889 | 8 |  |  |  |
| 23 | 减震脚垫 foot pad | 2 |  |  |  |
| 22 | 内六角花形盘头螺钉M6X50 GB-T2672 | 6 | 52 | 手柄管 handle | 1 |
| 21 | 弹簧垫圈￠8 spring washer | 4 | 51 | 手柄套 handle coat | 1 |
| 20 | 平垫圈_A级￠8 plain washer | 4 | 50 | 枪柄组件（长枪型）spray gun merge | 1 |
| 19 | 瓶嘴堵塞 blocking pourer | 1 | 49 | 警告标贴 waming label | 1 |
| 18 | 瓶嘴pourer | 1 | 48 | 莲花喷头挂架 lotous holder | 1 |
| 17 | 清洁液软管 detergent pipe | 1 | 47 | 塑料珠 string ball | 1 |
| 16 | 清洁壶标贴 bottle sticker | 1 | 46 | 弹性绳 string | 1 |
| 15 | 清洁壶 detergent bottle | 1 | 45 | 快速莲花喷头组件quick turbo merge | 1 |
| 14 | 清洁壶盖 bottle cover | 1 | 44 | 加长快速枪杆组件 quick lance merge | 1 |
| 13 | 枪托组件 gun stock | 1 | 43 | 铭牌 nameplate | 1 |
| 12 | 六角法兰面螺栓加大系列B级-M8X50 | 1 | 42 | 枪挂架 gun hook | 1 |
| 11 | 六角法兰面螺栓加大系列B级-M8X40 | 4 | 41 | 内六角花形盘头螺钉M6X45 GB-T2672 | 2 |
| 10 | 放油指示标贴 release oil sticker | 1 | 40 | 挂管钩 hose hook | 1 |
| 9 | 卧式引擎座 horizontal engine base | 1 | 39 | 高压管 high pressure hose | 1 |
| 8 | 引擎座标贴 engine base sticker | 1 | 38 | 喷嘴固定套 nozzle sheath | 5 |
| 7 | 安装指示标贴 assembling sticker | 1 | 37 | 低压快速喷嘴组件 l-q- nozzle assy | 1 |
| 6 | 内六角圆柱头螺钉M8X45 GB-T70 | 4 | 36 | 40°快速喷嘴组件q- nozzle assy | 1 |
| 5 | 卧式汽油机（隆鑫）horizontal engine loncin | 1 | 35 | 25°快速喷嘴组件 q- nozzle assy | 1 |
| 4 | 高压泵组件 pump assy | 1 | 34 | 15°快速喷嘴组件 q- nozzle assy | 1 |
| 3 | 进水螺纹护套 inler cap | 1 | 33 | 0°快速喷嘴组件 q- nozzle assy | 1 |
| 2 | 出水螺纹护套 outler cap | 1 | 32 | 主标贴 logo sticker | 1 |
| 1 | 进水螺母组件 water inler adapter | 1 | 31 | 8mm单头弹扣 locking piece | 2 |
| 序号 | 名称NAME | 数量 | 序号 | 名称NAME | 数量 |

汽油机结构图及明细

machine structure drawing and BOM

泵的爆照图及明细

Explosive drawing of pump and gun

| 1平键 key | 35 单向阀堵头 check valve plug |
| --- | --- |
| 2轴用弹性挡圈D35 GB894 | 36 O形圈10.6X1.8/80HS/ROHS O-ring |
| 3油缸oil tank | 37 温控器罩 temp valve cap |
| 4 FB型骨架油封52*35*8 oil seal | 38温控器G1/8-63℃ temp relief valve |
| 5推力轴承-￠42 thrust bearing | 39十字槽盘头螺钉M5X6 GB-T818 |
| 6斜盘_11.5度 wobble plate | 40 O形圈4.8X1.8/90-95HS/ROHS O-ring |
| 7推力轴承-￠25 thrust bearing | 41开口挡圈5X8X1.3 split washer |
| 8油缸组件 oil tank assy | 42溢流阀垫片 bypass valve washer |
| 9单向阀堵头（铝）valve plug (al) | 43溢流阀弹簧 bypass valve spring |
| 10 O形圈10.6X1.8 O-ring | 44溢流阀阀杆 bypass valve bolt |
| 11 O形圈70X2/70HS/ROHS O-ring | 45 O形圈10X1.8/80HS/ROHS O-ring |
| 12油封-12X20X5 oil seal | 46溢流阀堵头bypass valve plug |
| 13油封支架 oil seal holder | 47 O形圈12.5X1.8/80~85HS/ROHS O-ring |
| 14泵体组件 piston guider assy | 48溢流阀组件bypass valve assy |
| 15内六角圆柱头螺钉M8X25 screw | 49溢流阀螺帽 bypass valve nt |
| 16泵体 piston guider | 50溢流阀阀座 bypass valve base |
| 17柱塞 piston | 51内六角圆柱头螺钉M8X45 screw |
| 18柱塞弹簧piston spring | 52堵头G1/8 plug |
| 19柱塞卡盘 piston chuck | 53泵盖 pump cover |
| 20柱塞组件 piston assy | 54单向阀组件 check valve assy |
| 21出水管组件-25x15 outlet tube assy | 55单向阀阀座 |
| 22出水管-25x15 outlet tube | 56 O形圈8.5X1.8/80HS/ROHS O-ring |
| 23 O形圈10X1.8/80HS/ROHS O-ring | 57单向阀阀片check valve block |
| 24虹吸喷嘴 siphon nozzle | 58单向阀弹簧 check valve spring |
| 25总单向阀弹簧 chief valve spring | 59单向阀支架 check valve holder |
| 26总单向阀阀芯 chief valve core | 60 水封water sealing kit |
| 27 O形圈4.5X2/80HS/ROHS O-ring | 61主水封12X20X5-2400PSI direct seal washer |
| 28锥形弹簧 cone spring | 62水封垫圈 water seal washer |
| 29不锈钢球￠5 ball | 63 O形圈22X1_5/80HS/ROHS O-ring |
| 30 O形圈4X1.5/80HS/ROHS O-ring | 64水封座 water seal seat |
| 31 虹吸头-23mm suction nipple | 65副水封12X20X4 assistant seal washer |
| 32 虹吸头组件 suction nipple assy | 66 O形圈4.8X1.8/80HS/ROHS O-ring |
| 33 单向阀堵头组件 check valve plug assy | 67进水管-50 inlet tube |
| 34 YLP1530铝泵盖组件 pump head lv assy |  |

枪装配图及明细

Structure drawing of pump and gun

| 1长旋转螺母（铜）l rotate nut(cu) | 35 右枪壳 right gun housing |
| --- | --- |
| 2 O形圈9.8X2.4/80HS/ROHS O-ring | 36 扳机销 trigger pin |
| 3公接头（铜）core adapter (cu) | 37 枪阀组件（￠14）valve assy |
| 4 十字槽盘头自攻螺钉ST3.5X13F screw | 38扳机 trigger |
| 5内管 inside pipe | 39扳机锁扣 trigger lock button |
| 6枪柄杆右盖 r-housing lance | 40 握手软垫 handle cushion |
| 7扳机弹簧 trigger spring | 41枪柄杆左盖 l-housing lance |
| 8扳机圆垫片 trigger gasket | 42左枪壳 left gun housing |

第一章 适用范围

CHAPTER 1   USING BOUND

本系列清洗机是由三缸柱塞斜盘泵及汽油机配套组成。它具有结构紧凑，移动方便，调压方便，流量稳定，使用可靠等优点。主要用于军用机械、园林绿化、卫生防疫领域的清洗和消毒，适用于电源不方便处野外作业。

Three-tank pump and motor constituted this series machines, which have many advantages such as logic construction, easy voltage-adjustment, stable flow and so on. It can be used for cleaning cars, building equipment, machines and ground.

第二章 规格型号及技术参数

CHAPTER2   STANDARD AND TECHNICAL PARAMETER

| 型号Model No. 参数 parameter | YLQ7711E-200 | YLQ7711E-250 |
| --- | --- | --- |
| 额定输入功率 Rated input power | 4.1kW | 4.9kW |
| 外壳防护等级 Degree of protection | IP44 | IP44 |
| 额定压力 Rated pressure | 17MPa | 21MPa |
| 最大压力 Max. pressure | 20MPa | 25MPa |
| 额定流量 Rated flow | 8.7 l/min | 10.2 l/min |
| 额定进水温度 Rated temperature | 40 ℃ | 40 ℃ |
| 重量 Weight | 39.5 | 39.5 |

第三章 安全注意事项

CHAPTER 3   CAUTIONS

- 警告：严禁儿童及未经培训的人操作。

Caution: DO NOT allow children or untrained persons to use the appliance.

- 如果需要使用延长电源线，电源线插座必须是防水的。

If an extension cord is used, the plug and socket must be of watertight construction.

- 当重要部件（如：安全装置、高压胶管、扳机枪）损坏时，禁止使用本设备。

Caution:  Do not use the appliance if a supply cord or important parts of the appliance are damaged, e.g. safety devices, high pressure hoses, trigger gun.

- 警告：高压胶管、装配部件对设备的安全性非常重要，请务必使用由我公司推荐的胶管、装配件。喷头一定要使用我公司提供的配套喷头。

Caution: High pressure hoses, fittings and couplings are important for safety of the appliance. Use only hoses, fittings and couplings recommended by the manufacturer.

- 警告：高压水流使用不当会产生危险，不能将水流直接对人、运转中的电动设备及本设备自身冲洗，不要用水流直接对自己或他人借以清洁衣服或鞋子。

Caution: High pressure jets can be dangerous if subject to misuse. The jet must not be directed at persons, live electrical equipment or the appliance itself. Do not direct the jet against yourself or others in order to clean clothes or foot-wear.

- 不要在没有穿防护服的人群中使用本机器。

Caution: Do not use the appliance within range of persons unless they wear protective clothing.

- 为保证设备的安全使用，应使用我公司提供或认可原始部件。

Caution: To ensure appliance safety, use only original spare parts from the manufacturer or approved by the manufacturer.

- 警告：本清洗机制造商已推荐了专用清洗剂，使用其它的清洗剂或化学剂会对清洗机的安全造成影响。

WARNING: This appliance has been designed for use with the cleaning agent supplied or recommended by the manufacturer. The use of other cleaning agents or chemicals may adversely affect the safety of the appliance.

- 警告：机器噪音可能伤害人耳，应使用耳塞防护.

Appropriate ear protection is suggested to be used during operation.

- 严禁儿童及没有人陪同的有身体缺陷或智力障碍的人使用本清洗机。

This appliance is not intended for use by children or other persons without assistance or supervision if their physical, sensory or 关mental capabilities prevent them from using it safely. Children should be supervised to ensure that they do not play with the appliance. Before starting up your machine please check it carefully for any defects. If you find any, do not start up your machine and contact your distributor.

- 对清洗机进行维修时，必须断开电源

Disconnect from the electrical power supply before carrying out user maintenance.

- 设备工作时必须有人照管。

This appliance must be observed during operation.

- 对安全装置不得随意变动。

DO NOT change any safety devices.

- 设备在进行维护保养之前请确保发动机水平停止。

Ensure engine is stopped and level before performing anymaintenance or service.

- 为了您的安全,请您务必按以上要求使用本机器。

Please according to above items mentioned to operate appliance.

第四章 使用及保养

CHAPTER 5   USING AND MAINTAIN

- 开箱后根据装箱单一一核对清楚，附备件及文件资料要妥善保管。

Please check if everything is in the carton according to the packing list after opening and please keep accessories and documents.

- 将手柄垂直插入机架上，如下图1所示，待听到机架上弹片弹出声响后，安装完成。请见整机结构图明细表的第31项。

Insert the handle vertical racks, as shown in the figure below, after being heard on chassis shrapnel pop-up, the installation is complete. (Refer to item 31 in BOM of structure drawing.)

图1

- 本机使用环境为海拔4000米以下，周围环境温度摄氏零下10℃--摄氏40℃（注意水源应满足进水要求）。环境湿度98%以下，距离易燃易爆源3米以外使用。

Working condition: Altitude below 3500m; temperature -0℃-- 40℃; Humidity less than 98%.

- 开机前需向汽油机内加入燃油（如图2-4）。每次使用前都应检查润滑油的位置，如低于油位线，要及时添加。具体操作方法：如（图2）首先旋下汽油机上面的量油尺N，加到油标的最高油位为止（注意不能加满）。如油量少于最低油位或需更换机油应是旋下汽油机下面的放油塞L，待费油放尽再旋好加机油(同前)。

Please add oil up to the maximum oil level of oil-ruler. 。Please check it before using every time and add oil if it is below the minimum oil level（drwing2-5）

图2             图3

- 推动动燃料开关“U”，把汽油机开关调整开启状态“O” ，汽油机启动（如图4 “P-U”）

Turn fuel shut-off valve to ON position (if equipped).Turn engine on/off switch to ON position (if equipped).Start engine as follows: Cold engine: Place throttle control midway between SLOW and FAST positions. Place choke control into ON position.Warm engine: Place throttle control midway between SLOW and FAST positions. Return choke to OFF position as soon as engine starts. A warm engine usually does not require choke on.Retractable Start: Slowly pull starter handle until just past compression-STOP! Return starter handle; fi rmly pull straight out to avoid excessive rope wear from starter rope guide. Electric Start: Activate starter switch. Release switch as soon as engine starts. If starter does not turn engine over, shut off starter immediately. Do not make further attempts to start engine until condition is corrected. Do not jump start. See your Kohler authorized dealer for trouble analysis.Gradually return choke control to OFF position after engine starts and warms up. Engine/equipment may be operated during warm up period, but it may be necessary to leave choke partially on until engine warms up.

图4

- 高压胶管和进水胶管分别同出水口、进水口连接并拧紧，将进水胶管的与水源连接端放入干净水箱中或与水龙头直接连接（具体可视机器配置而定）。

High pressure hose be connected to water-outlet and water inlet hose be connected to water-inlet.

- 若用水箱供水，水质必须清洁，水箱高度应高出进水口1米以上，定期对水箱及滤网进行清洗。

Make sure the water we use should be clean. Clean the water tank and filter after every 50 hours of operation.

- 清洗工作时，机具压力以不超过额定压力。

Pressure should be less than rated pressure when working.

- 本机最高工作压力出厂时已调定，请用户不要随意自行调节。

Working pressure has been adjusted before delivery to customers, please do not change it.

- 新机使用5小时后,应更换泵内机油,以后累计使用50小时后或1年应重新更换机油 （图2）

Change the machine oil after 5 hours for the first and every 50 hours or 1 year after that..

- 累计使用100小时后或1年，清洗过滤器，更换滤芯.操作方法如（图5）旋转翼形螺帽，拆分过滤器清洗（图5）。同时定期清洗或更换喷枪喷嘴水垢等.

Clean the dual-element air cleaner for every 100 Hours or Annually1(drawing5) Rotary wing nut,and split. replace dual-element air cleaner element.  And clean the filter in water-inlet hose also

图5

- 长期不用时，应将进水管离开供水源,开机脱水运转半分钟排尽泵内剩水。

If you will not use this machine for a long time, please make sure that hose and pump are dry.

第五章．装箱单

CHAPTER 6  PACKING LIST

| 序号 No. | 名称 Name | 数量 Quantity | 规格 Specification | 备注 Remark |
| --- | --- | --- | --- | --- |
| 1 | 清洗机 pressure washer | 1 |  |  |
| 2 | 使用说明书 Manual | 2 |  |  |
| 3 | 喷枪+枪杆 Spray gun and lance | 2 |  |  |
| 4 | 高压管组件 High-pressure hose ass’y | 1 |  |  |
| 5 | 进水管组件 Water inlet ass’y | 1 |  |  |

第六章．故障分析与排除

CHAPTER 7  ANALYSING AND REMOVING OF MULFUNCTION

| 故障 Problem | 故障原因 Possible causes | 排除方法 Remedy |
| --- | --- | --- |
| 发动机不起动 Motor fails to start | - 未插电源 Machine not plugged in - 电源插槽有问题 Defective socket - 保险丝已熔断 Fuse has blown - 电源延长线断裂 Defective extension cord - 漏电保护器断开 GFCI is in “OFF” positon. | - 插好插头 Plug in machine - 换个插孔 Try another socket. - 换保险丝。关掉其它电器 Replace fuse. Switch off other machines. - 不用电源延长线试试。 Try without the extension cord. - 打开漏电保护器 Press GFCI to "ON" position |
| 电机在运转，但泵无排量或量小 Machine starts, but no water comes out | - 泵/高压管或附件等管道被冻结 Pump/hoses or accessory frozen - 没有水供应 No water supply - 过滤器被堵塞 Water filter blocked 4. 喷嘴被堵塞 Nozzle blocked | 1. 等待管道解冻 Wait for pump/hoses or accessory to thaw. 2. 连接水源 Connect inlet water. 3. 清洁过滤器 Clean the filter - 清洁喷嘴 Clean the nozzle |
| 漏水或漏油 Water leakage or oil leakage | 1. 螺纹连接处,螺纹未拧紧 Screw is loose. 2. O型圈或垫圈损坏 O-ring or washer is broken | 1. 旋紧螺纹 Tighten screw. 2. 更换O型圈或垫圈 Change O-ring or washer |
| 泵压力不够 Pump does reach working pressure | 1. 喷嘴损坏 Nozzle worn 2. 过滤器淤塞 Water filter fouled 3. 水源压力太低 Water supply pressure low 4. 管路中有空气 Air being sucked into system 5. 空气进入泵 Air in pump | - 更换喷嘴 Replace nozzle - 清洁过滤器 Clean filter - 旋紧水源接头 Turn on water supply tap fully - 检查并拧紧管路连接处 Check tightness of hose fittings - 关掉清洗机，持续按压和放开喷枪的扳机，直到稳定的水源喷出，重新打开电源。 Switch off the appliance and keep depressing and releasing the gun trigger until the water comes out in a steady flow. Switch the appliance back on again. |

上海亿力电器有限公司

Shanghai yili electric Co., Ltd

地址：上海闵行区颛桥镇向阳路1358号

电话：+86 21 64892787-1046

传真：+86 21 64895050

邮编：201108

E-mail:：

Http：//www.yili-biz.com